Virtual i-O i-glasses! Tracker Software Development Toolkit

Copyright 1995 Virtual i-O

All Rights Reserved

Version 1.0

July 31, 2026

Installation

Make a directory on your hard drive called vio. Copy the directory VIODKCD to vio.  Hereafter, C: refers to the drive where you created vio.

Everything you need to know about programming the tracker is included in these libraries.  To recompile and work with these files, set the environment variable SOFT to point to where these files reside.  For example, if these files are located in C:\vio, then type: set SOFT=C:\vio.

If you include %SOFT%\batch in your path, you can have access to handy batch files to quickly move from one library directory to another.  For example, if you type imath, you'll be placed in %SOFT%\src\imath1.  This was done to make DOS more UNIX-like (aliases).

You can also cd to C:\vio and type setenv, which will setup the environment variable SOFT and add %SOFT%\batch to the path.

Usage

To make a library, simply type m, which calls wmake -f watcom.mak.

To update a library (make it and copy it to %SOFT%\libs) type m update.

To relink a higher level library tester or application, type l.

To clean a directory, type m clean.

To clean and update a library, type m remake.

Makefiles

This development system uses a generic makefile, which is located in %SOFT%\make.  All frequently used information is placed in this file so individual library makefiles are trivially simple (they include %SOFT%\make\genwat.mak).  If you use stack based arguments and/or don’t have DOS/4GW Pro, you’ll have to make changes to genwat.mak.

Stack based compilation for Watcom

To use stack based arguments with Watcom, uncomment the USE_STACK_ARGS line in  SOFT%\make\genwat.mak (remove the #), then remake the entire library. Type src, then type makeall  (calls batch files in %SOFT%\batch).

Building the libraries and testers with DOS/4GW Professional

If you have DOS/4GW Professional (from Tenberry Systems), you must either set the environment variable use4gwpro (set use4gwpro=1) or modify %SOFT%\make\genwat.mak by uncommenting the line USE_4GWPRO (remove the #).

Stereoscopic 3D rendering

Examine the code in src\grid1. Check out test.c: this code shows how to set up the tracker and draw stereo 3D in interleaved mode.  The graphics code is located in src\vgfx1 for direct video memory rendering, and src\dramgfx1 for off screen rendering and interleaved copying.  Press ‘b’ to toggle the background color.  Press 's' to toggle stereo 3D and 'd' to toggle DRAM rendering (unless you're using a Pentium, DRAM rendering should be much slower than direct display memory rendering.  That's *this* example only.  Full texture mapped screens with transparency effects will probably run faster when rendering in DRAM).

SciTech's direct SVGA linear addressed graphics access should make this quite a bit easier (320x400, 256 to 64k color modes, addressed linearly, and not through slow VGA hardware (no outs, of course, either).  Contact SciTech for more info.

Integer math

All math is now in 14 bit fixed point (1<<14 = 1.0).  The yaw computation now does not require square root and has worst case error of 12 thousandths of a degree with 4 thousandths of a degree average case (when compared to floating point).  If you do your own integer math, it is extremely important to have an accurate yaw computation.  If you can do better, please let us know.  The lookup tables used here are 256 entries for sine and 256 entries for arctanget.  This uses 2048 bytes.  The lookup tables are 28 bit fixed point and the entries are interpolated when looked up.

See src\convert1\convert.c for the math required to perform the angle computations (it’s fast, short, and simple).

Filtering

The filtering method has changed to adaptive filtering (and will change again).  I am currently experimenting with filtering in this library (you'll see it in src\convert1\convert.c.  The filter code is in src\filter1\filter.c (currently working as a simple running average, although the code is a general Finite Impulse Response (FIR) filter library.  An Infinite Impulse Response (IIR) interpolating filter is also included).  If your application does additional filtering (as in src\convert1\convert.c), you must have an option to turn it on and off.

For now, don't set the tracker filter settings in your application.  Use tcal to set the filters.  We'll provide a user preferences tool with the tracker to set filter values (tracker.exe in the apps\trackman directory).

If you use tcal, DO NOT ATTEMPT TO CALIBRATE THE TRACKER.  Factory calibration requires hardware jigs and cannot be done reliably by hand.  We are providing this code so you can see how everything works (it's an open system).  Once factory calibrated, the tracker should not need to be calibrated again.

16-bit DOS Compilation (Borland C/C++ compiler. Tested with version 3.1)

For the purpose of creating real-mode drivers, the following modules have been modified to allow compilation under Borland C/C++ 3.1 (or later) for 16-bit 386 DOS real-mode:

imath.c

vector.c

timer.c

filter.c

convert.c

serial.c

tracker.c

These are all the files that you need to write a real-mode driver. The vector1 and filter1 libraries are optional (vector1.h is needed by other libraries, though).

Examine %SOFT%\apps\simple for a single header and single library example that uses the bare essentials.

For a Borland 16-bit real-mode version, see %SOFT%\src\apps\bsimple. The library vstrack1.lib and header vstrack1.h can be used to create a real-mode driver.

To make a Borland .obj file, type b, which calls make -f borland.mak.

To remake all Borland files, type bmakeall (in %SOFT%\src).

Real-mode driver examples

Example real-mode drivers for Raven’s Heretic and LucasArts’ Dark Forces can be found in apps\htdrv and apps\dark respectively.  A better way to handle drivers would be to use 32-bit protected mode DLLs.  This is more elegant and will have higher performance as real/protected mode swapping won’t be necessary.  Contact Tenberry Systems for more information.

Notes

The tracker specification is finalized (for a 1.0 version).  Thus, if you follow the spec, you don't need to use any of this code directly.  If you find a difference in the spec and the code, please let us know.

Do not use any of the code that reads and writes to tracker memory.  This will change and is not otherwise documented.  To set filter values and mode settings, use tracker mode commands (not tracker read/write commands).

If you get an error communicating with the tracker, send a "!\r" string to the tracker until you get an 'O'.

If you keep sending just 'S', the tracker may never respond until reset with a "!\r" or other ! command.  The examples call resetTracker(), which performs this operation.

Virtual i-O: 206.382.7410

July 31, 2026